SmartShield IPS

Step-by-Step Running Guide

For Linux + Docker  •  Demo Recording Edition

Written in plain language — no prior experience needed

What you will do in this guide: Prepare → Run → Simulate Attacks → See Alerts → Record Demo

# What Is SmartShield?

Think of SmartShield like a smart security guard for a factory. In a real factory, machines talk to each other through a network — PLCs (the computers that control motors, valves, conveyors) send messages back and forth constantly. SmartShield sits quietly and watches all these messages. If someone tries to attack the machines — like sending fake commands, flooding the network, or trying to stop a PLC — SmartShield catches it, logs it, and can even block the attacker.

Your system has two parts:

- SmartShield IPS: The brain. Runs as a Python program inside Docker.

- Web interface: A webpage you open in your browser to see alerts and what is happening in real time.

ℹ For your demo You will run SmartShield on your own Linux PC, pretend your PC is a factory network, simulate attacks (using simple scripts), and show the alerts appearing live on the web interface — exactly what a doctor wants to see.

# What You Need Before Starting

## Check your Linux has these installed

Open a terminal and run each command below. If it prints a version number, you are good. If it says "command not found", install it.

# Check Docker

docker --version

# Should show something like: Docker version 24.0.5

# Check Docker Compose

docker compose version

# Should show something like: Docker Compose version v2.20.0

# Check Python

python3 --version

# Should show: Python 3.10 or higher

# Check Git (optional but useful)

git --version

If Docker is not installed:

sudo apt update

sudo apt install -y docker.io docker-compose-plugin

sudo systemctl start docker

sudo systemctl enable docker

# Allow your user to run Docker without sudo

sudo usermod -aG docker $USER

# IMPORTANT: Log out and log back in after this command

If Python is missing:

sudo apt install -y python3 python3-pip

## Install the attack simulation tools

To make a good demo, you need tools that can send fake attack traffic. Install them:

sudo apt install -y nmap hping3 netcat-openbsd

# Install Python Modbus simulator (to simulate PLC attacks)

pip3 install pymodbus scapy --break-system-packages

💡 What these tools do nmap — scans ports, like a burglar checking which windows are open in a house hping3 — sends many packets very fast, like flooding a factory gate with trucks pymodbus — speaks the Modbus language that PLCs use, so we can fake PLC commands netcat — sends raw data to any port, like knocking on a door very fast

# Steps to Run SmartShield

STEP 1 Get the Project Code Ready You already have the zip file (SmartShield-Factory.zip) from the previous session. Put it somewhere easy to find, like your home folder. Open a terminal and run:

# Go to your home folder

cd ~

# If you have the zip file, extract it:

unzip SmartShield-Factory.zip

# Go into the project folder

cd SmartShield-Factory

# See what is inside

ls

# You should see folders like: modules/, config/, docker/, webinterface/, etc.

✅ Good sign You should see a list of folders. If you see "modules config docker webinterface zeek-scripts" you are in the right place.

STEP 2 Set Up the Configuration Files You need to tell SmartShield which IPs to protect and which are authorized. For your demo on a local PC, we will use fake local IP addresses.

2a. Create a simple OT devices protection list:

# Open the file with a text editor

nano config/ot_protected_devices.conf

# Add these lines (pretend these are your factory PLCs):

# (Press Ctrl+O to save, Ctrl+X to exit nano)

# For demo: protect these fake PLC IPs

172.20.0.10   # Fake PLC-01

172.20.0.11   # Fake PLC-02

172.20.0.20   # Fake HMI

2b. Leave the Modbus masters file empty for now

An empty file means SmartShield will detect ALL Modbus writes as unauthorized — perfect for showing attacks in the demo.

# Just make sure the file exists (it already does)

ls config/modbus_authorized_masters.conf

2c. Set your web interface password:

# Create the .env file that Docker will use

cat > docker/.env << 'EOF'

SMARTSHIELD_INTERFACE=eth0

SMARTSHIELD_WEB_USER=admin

SMARTSHIELD_WEB_PASSWORD=Demo1234567!

SMARTSHIELD_SECRET_KEY=abc123def456ghi789jkl012mno345pqr678

EOF

echo "Done! .env file created."

⚠ Important for the demo The interface name (eth0) does not matter much for a local demo — SmartShield will read from PCAP or network. For live capture on your machine, replace eth0 with your actual network interface name. To find your interface name, run: ip link show

STEP 3 Build the Docker Image This turns the code into a ready-to-run container. You only do this once (or when code changes). This may take 5–15 minutes the first time. Go make tea. ☕

# Make sure you are in the project root folder

cd ~/SmartShield-Factory

# Build the image (this takes a while first time)

docker build -f docker/Dockerfile -t smartshield_gp:latest .

# When it finishes you should see:

# Successfully tagged smartshield_gp:latest

If the build fails with a network error:

# Try again — sometimes package downloads timeout

docker build -f docker/Dockerfile -t smartshield_gp:latest . --no-cache

Verify the image was built:

docker images | grep smartshield

# You should see a line like:

# smartshield_gp   latest   abc123...   5 minutes ago   1.2GB

STEP 4 Create the Data Folders SmartShield needs folders on your computer to save alerts and data between restarts.

# Create the folders where SmartShield saves its data

sudo mkdir -p /var/lib/smartshield/redis

sudo mkdir -p /var/lib/smartshield/output

# Give Docker permission to write to them

sudo chmod 777 /var/lib/smartshield/redis

sudo chmod 777 /var/lib/smartshield/output

# Verify they exist

ls /var/lib/smartshield/

# Should show: output  redis

STEP 5 Start SmartShield Now start all the containers with one command. This starts SmartShield, Redis (the database), and the web interface.

# Go into the docker folder

cd ~/SmartShield-Factory

# Start everything

docker compose -f docker/docker-compose.factory.yml up -d

# The -d means "run in background" so your terminal stays usable

# Wait 10 seconds, then check if they are running

sleep 10

docker compose -f docker/docker-compose.factory.yml ps

You should see something like this:

NAME                 STATUS          PORTS

smartshield          Up 15 seconds

smartshield-redis    Up 15 seconds   127.0.0.1:6379->6379/tcp

smartshield-web      Up 15 seconds   0.0.0.0:55000->55000/tcp

💡 If a container shows "Exiting" or "Error" Run this to see what went wrong: docker logs smartshield docker logs smartshield-redis The most common problem is the docker/.env file missing — check Step 2.

STEP 6 Open the Web Interface The web interface is where you will see all alerts during the demo. Open it in your browser.

# Open your browser and go to:

http://localhost:55000

# Or if your Linux has a desktop, run:

xdg-open http://localhost:55000

You will see a login page. Enter:

| Field | Value |
| --- | --- |
| Username | admin |
| Password | Demo1234567! (or whatever you set in Step 2) |

✅ You are in! After login you should see the SmartShield dashboard. At first it will look empty — that is normal. Alerts appear after attacks are detected. Keep this browser tab open for the demo.

To see live logs while it runs (open a second terminal):

docker logs smartshield -f

# The -f means "follow" — new lines appear in real time

# Press Ctrl+C to stop watching logs

# How to Test It — Simulating Attacks

For your demo, you will run fake attacks on your own computer. SmartShield is watching the network and will detect them. Think of it like a fire drill — you set off the alarm on purpose to prove the system works.

⚠ These attacks are safe on your own PC All the commands below target localhost (127.0.0.1) or your own machine. NEVER run these on a real factory network or someone else's network. They are harmless on your own PC and will not damage anything.

## Attack 1 — Port Scan (Simulates a Hacker Discovering the Network)

A port scan is like a burglar walking along a street, knocking on every door to see which ones open. SmartShield will see many connections and flag it.

In a NEW terminal, run:

# Simple port scan against your own machine

# This simulates an attacker discovering Modbus port 502

nmap -sS -p 502,102,20000 127.0.0.1

# Aggressive scan — more noise, easier for SmartShield to detect

nmap -sV -p 1-1000 127.0.0.1

# Scan targeting OT ports specifically

nmap -p 502,102,20000,4840,44818,34964 --open 127.0.0.1

Then check the web interface — you should see a new alert appear within 1–2 minutes.

## Attack 2 — Modbus Connection Flood (Simulates Flooding a PLC)

This is like someone calling a PLC's phone number thousands of times per minute so the real operator cannot get through. SmartShield will count the connections and alert when the threshold is exceeded.

First, run a Modbus server so there is something to attack:

# Terminal 1 — Start a fake Modbus PLC server

python3 - << 'PYEOF'

from pymodbus.server import StartTcpServer

from pymodbus.datastore import ModbusSlaveContext, ModbusServerContext

from pymodbus.datastore import ModbusSequentialDataBlock

import logging

logging.basicConfig()

store = ModbusSlaveContext(

di=ModbusSequentialDataBlock(0, [17]*100),

co=ModbusSequentialDataBlock(0, [17]*100),

hr=ModbusSequentialDataBlock(0, [17]*100),

ir=ModbusSequentialDataBlock(0, [17]*100)

)

context = ModbusServerContext(slaves=store, single=True)

print("Fake PLC running on port 502...")

StartTcpServer(context=context, address=("0.0.0.0", 502))

PYEOF

Then in a second terminal, run the flood attack:

# Terminal 2 — Flood the fake PLC with connections

python3 - << 'PYEOF'

from pymodbus.client import ModbusTcpClient

import time

print("Starting Modbus flood attack...")

for i in range(50):

try:

c = ModbusTcpClient("127.0.0.1", port=502)

c.connect()

c.read_coils(0, 10)   # Read coils

c.close()

except Exception as e:

pass

if i % 10 == 0:

print(f"Sent {i} connections...")

print("Done! Check SmartShield web interface for alerts.")

PYEOF

💡 What to show in the demo Go to the web interface → Analysis page. You should see the attacker IP (127.0.0.1) appear with evidence of a Modbus connection flood. The threat level will be HIGH.

## Attack 3 — Modbus Unauthorized Write (Simulates Tampering with PLC)

This is the most dangerous attack in real factories. An unauthorized computer sends a WRITE command to a PLC — changing a coil value that could open a valve or stop a motor. SmartShield detects writes from non-authorized sources.

# Make sure the fake PLC from Attack 2 is still running in Terminal 1

# In Terminal 2, run an unauthorized write:

python3 - << 'PYEOF'

from pymodbus.client import ModbusTcpClient

print("Attempting unauthorized Modbus coil write...")

client = ModbusTcpClient("127.0.0.1", port=502)

if client.connect():

# Write Single Coil — function code 5

# In a real factory this could open a valve or stop a motor

result = client.write_coil(0, True)

print(f"Write coil result: {result}")

# Write Multiple Coils — function code 15

result = client.write_coils(0, [True, False, True, True, False])

print(f"Write multiple coils: {result}")

# Write Single Register — function code 6

result = client.write_register(1, 9999)

print(f"Write register: {result}")

client.close()

print("Attack complete. Check SmartShield!")

else:

print("Could not connect. Is the fake PLC running?")

PYEOF

🛑 What this shows the doctor SmartShield detects UNAUTHORIZED WRITES to PLCs — the most dangerous ICS attack. The evidence type will be: OT_MODBUS_WRITE_COILS with CRITICAL threat level. In a real factory, this would trigger an alert to the SOC (security operations center).

## Attack 4 — TCP SYN Flood (Simulates DoS on OT Network)

A flood attack overwhelms a PLC or network device so it stops responding. Like calling an emergency phone line 1000 times per second so no one else can call.

# Run a SYN flood on port 502 (Modbus) - uses hping3

# This simulates a Denial of Service attack on a PLC

sudo hping3 -S -p 502 --flood 127.0.0.1

# Let it run for 10 seconds, then press Ctrl+C to stop

# SmartShield will detect the flood from the connection count spike

## Attack 5 — Quick Nmap Service Scan (Shows Network Discovery)

Attackers first scan to find what is on the network before attacking. This simulates reconnaissance — the first stage of any cyberattack on a factory.

# Service version detection scan — loudest type of scan

nmap -sV -O 127.0.0.1

# UDP scan targeting OT protocols

sudo nmap -sU -p 20000,34964,319,123 127.0.0.1

# Aggressive scan with script detection

nmap -A -p 502,102 127.0.0.1

ℹ When to run each attack for the demo Run attacks in this order for maximum visual impact: 1. Nmap scan first (easy to explain) 2. Modbus flood (dramatic spike in connections) 3. Unauthorized write (most serious — shows CRITICAL alert) 4. Show the web interface filling up with alerts 5. Show blocking.log to prove the attacker was blocked

# Seeing the Results

## In the Web Interface

Go to http://localhost:55000 in your browser. Here is what each page shows:

| Page | What it shows |
| --- | --- |
| Overview page (/) | Summary — how many profiles, how many alerts, when analysis started |
| Analysis (/analysis) | The main page — click any IP to see all evidence and attack details |
| Timeline | A chronological view of all events — great for showing the doctor |

## In the Terminal

See alerts in real time:

# Watch SmartShield logs live

docker logs smartshield -f

# See the alerts file

cat /var/lib/smartshield/output/alerts.json

# Watch new alerts appear (updates every second)

watch -n 1 "cat /var/lib/smartshield/output/alerts.json | tail -20"

# See what IPs are blocked

sudo iptables -L smartshieldBlocking -n 2>/dev/null || echo "blocking module not active"

# Count total alerts generated

docker exec smartshield-redis redis-cli KEYS "alert_*" | wc -l

# List all evidence stored in Redis

docker exec smartshield-redis redis-cli KEYS "evidence_*" | head -20

## Quick Status Check Command

Run this any time to see a full health check:

# Full status in one command

echo "=== Containers ===" && docker compose -f docker/docker-compose.factory.yml ps

echo "=== Alerts count ===" && docker exec smartshield-redis redis-cli DBSIZE

echo "=== Output files ===" && ls -lh /var/lib/smartshield/output/

echo "=== Redis persisting? ===" && docker exec smartshield-redis redis-cli INFO persistence | grep aof_enabled

# Stopping and Restarting

## To stop SmartShield (data is saved)

docker compose -f docker/docker-compose.factory.yml down

# All your alerts and data are saved in /var/lib/smartshield/

# Redis writes to disk so nothing is lost

## To start again later

cd ~/SmartShield-Factory

docker compose -f docker/docker-compose.factory.yml up -d

# Wait 10 seconds then open browser at http://localhost:55000

# Your previous alerts will still be there

## To restart (after config changes)

docker compose -f docker/docker-compose.factory.yml restart

# Or restart just SmartShield:

docker restart smartshield

## To see logs at any time

docker logs smartshield --tail 50

docker logs smartshield-redis --tail 20

docker logs smartshield-web --tail 20

# Recording a Good Demo for Your Doctor

## Recommended Demo Script (8–10 minutes)

| Time | Section | What to say/show |
| --- | --- | --- |
| 0:00–1:00 | Introduction | Show the web interface (empty, clean). Explain what SmartShield is and where it sits in a factory network. |
| 1:00–2:00 | Start monitoring | Show the terminal with "docker logs smartshield -f" running. Explain Zeek is capturing traffic. |
| 2:00–3:30 | Attack 1: Port scan | Run nmap in the terminal. Switch to web interface — show a new IP profile appearing. |
| 3:30–5:00 | Attack 2: Modbus flood | Start fake PLC, run flood script. Show connection count rising in web interface. |
| 5:00–7:00 | Attack 3: Unauthorized write | Run write attack. Show CRITICAL alert appearing. Explain this would stop a motor or open a valve in real factory. |
| 7:00–8:00 | Show blocking | Run: sudo iptables -L smartshieldBlocking -n. Show the attacker IP was blocked. |
| 8:00–9:00 | Show OT-safe | Explain that if 192.168.10.10 (a PLC) were the source, it would NOT be blocked — factory safe. |
| 9:00–10:00 | SIEM export | Show alerts.json file. Explain this feeds into enterprise SIEM systems like Splunk or QRadar. |

## Screen Recording Tips

- Use OBS Studio or SimpleScreenRecorder on Linux. Install: sudo apt install obs-studio

- Record at 1080p if possible. The web interface looks much better in full screen.

- Have TWO windows side by side: terminal on left, browser (web interface) on right.

- Zoom into the alerts section when a CRITICAL alert appears — the red color is very visible.

- Narrate what you are doing: "I am now running a Modbus write attack to coil register 0..."

- Show the timestamp on alerts — it proves detection is real-time, not pre-recorded.

# Install screen recorder

sudo apt install simplescreenrecorder

# Or use OBS Studio

sudo apt install obs-studio

# To set up two windows side by side, right-click desktop → Tile Windows

# Or on GNOME: drag one window to the left edge, another to the right edge

## What to Say During the Demo

| Moment | What to say |
| --- | --- |
| When showing web UI | SmartShield uses a behavioral profiling approach — it builds a normal profile for every IP and alerts when behavior deviates from that profile. |
| When showing Modbus alert | Modbus is the most common industrial control protocol. An unauthorized write to a coil could open a safety valve or stop an assembly line. |
| When showing OT-safe blocking | In a real factory, blocking a PLC IP would stop production. SmartShield's OT-safe mechanism prevents this — we alert but never auto-block OT devices. |
| When showing SIEM export | The syslog export means this integrates with enterprise security systems like Splunk, IBM QRadar, or Wazuh without changing existing SOC workflows. |
| When showing Redis persistence | Even after a server restart, SmartShield remembers all evidence and blocked IPs. Critical for factory environments that run 24/7. |

# Common Problems and Fast Fixes

| Problem | Fix |
| --- | --- |
| "Permission denied" on Docker | Run: sudo usermod -aG docker $USER — then log out and log in again. |
| Web interface shows nothing at localhost:55000 | Wait 30 more seconds. Then run: docker ps — check smartshield-web is "Up". Then run: docker logs smartshield-web |
| Containers keep restarting / "Exiting" | Run: docker logs smartshield — look for error messages. Most common: missing .env file in docker/ folder. |
| No alerts appearing after attacks | SmartShield groups evidence into 1-hour time windows. Lower the threshold: edit smartshield.yaml, set evidence_detection_threshold: 0.05, then restart. |
| "Port 502 permission denied" | Port 502 needs root. Run: sudo python3 ... for the fake PLC server. |
| "pymodbus not found" | Run: pip3 install pymodbus --break-system-packages |
| nmap says "0 open ports" | The fake PLC needs to be running first (Terminal 1). Make sure it shows "Fake PLC running on port 502..." |
| Alerts.json file is empty | SmartShield may need 5–10 minutes to start up fully. Run: docker logs smartshield ¦ grep "Started" |

💡 If all else fails — full reset docker compose -f docker/docker-compose.factory.yml down sudo rm -rf /var/lib/smartshield/* sudo mkdir -p /var/lib/smartshield/redis /var/lib/smartshield/output sudo chmod 777 /var/lib/smartshield/redis /var/lib/smartshield/output docker compose -f docker/docker-compose.factory.yml up -d

# Cheat Sheet — All Commands in Order

Cut this page out and keep it next to your keyboard during the demo.

### Initial Setup (run once)

cd ~/SmartShield-Factory

sudo mkdir -p /var/lib/smartshield/{redis,output}

sudo chmod 777 /var/lib/smartshield/{redis,output}

docker build -f docker/Dockerfile -t smartshield_gp:latest .

### Start SmartShield

cd ~/SmartShield-Factory

docker compose -f docker/docker-compose.factory.yml up -d

sleep 10 && docker compose -f docker/docker-compose.factory.yml ps

# Then open browser: http://localhost:55000

### Demo Attacks (run in order)

# Attack 1 — Port scan

nmap -sV -p 502,102,20000 127.0.0.1

# Attack 2 — Start fake PLC (Terminal 1)

sudo python3 -c "

from pymodbus.server import StartTcpServer

from pymodbus.datastore import ModbusSlaveContext,ModbusServerContext,ModbusSequentialDataBlock

s=ModbusSlaveContext(co=ModbusSequentialDataBlock(0,[17]*100),hr=ModbusSequentialDataBlock(0,[17]*100),ir=ModbusSequentialDataBlock(0,[17]*100),di=ModbusSequentialDataBlock(0,[17]*100))

print("Fake PLC on port 502")

StartTcpServer(ModbusServerContext(slaves=s,single=True),address=("0.0.0.0",502))

"

# Attack 2 — Flood Modbus (Terminal 2)

python3 -c "

from pymodbus.client import ModbusTcpClient

[(__import__("time").sleep(0.1), ModbusTcpClient("127.0.0.1",port=502).connect()) for _ in range(50)]

"

# Attack 3 — Unauthorized write (Terminal 2)

python3 -c "

from pymodbus.client import ModbusTcpClient

c=ModbusTcpClient("127.0.0.1",port=502)

c.connect()

c.write_coil(0,True)

c.write_register(1,9999)

print("Done")

"

### Check Results

cat /var/lib/smartshield/output/alerts.json

docker logs smartshield --tail 30

sudo iptables -L smartshieldBlocking -n 2>/dev/null

### Stop Everything

docker compose -f docker/docker-compose.factory.yml down

# Kill the fake PLC: Ctrl+C in Terminal 1

SmartShield GP  —  Demo Running Guide 🎓